# FPC Suspension Strain Gauge — Research

Formula Slug | LV Onboarding | Research and feasibility calculations

# 1. Engineering Summary

A copper serpentine patterned on a polyimide flexible PCB could act as a bonded strain sensor on a suspension member. Its key advantage is integrated, repeatable fabrication and routing; its key limitations are copper’s low resistivity, strong temperature dependence, and uncertainty in strain transfer through the adhesive and flexible substrate. The calculations below indicate that a 350 Ω single-layer copper sensor is much larger than a conventional foil gauge. A lower-resistance copper FPC is therefore a sensible proof-of-concept to discuss with the team, not a validated replacement for a commercial gauge.

# 2. Operating Principle and Measurement

Strain ε = ΔL/L. Gauge factor GF = (ΔR/R)/ε; a metallic gauge often has GF near 2, but the effective GF of a bonded copper-on-polyimide design must be calibrated. For an illustrative 350 Ω gauge at 500 microstrain and GF = 2: ΔR = GF × ε × R = 2 × 0.0005 × 350 = 0.35 Ω. The 500 microstrain value is an example, not a verified suspension loading condition.

A quarter Wheatstone bridge with one active gauge gives approximately |Vout|/Vex ≈ |ΔR|/(4R) for small changes. A half or full bridge may improve sensitivity or temperature rejection, but only when gauge orientation and loading justify the arrangement. Under pure axial tension, gauges on opposite sides of a tube generally both experience tensile strain; opposite-sign strain is expected for bending about the relevant axis.

# 3. Existing FSAE-Style Approach and FPC Tradeoffs

A conventional suspension-load measurement approach bonds commercial foil gauges to a prepared pushrod, pullrod, or suspension tube, then reads the bridge using low-noise instrumentation and a DAQ. Gauge orientation depends on whether the objective is axial force, bending, or a combination. Specific named-team implementations should be cited before attributing them to a university; this document does not make unsupported team-specific claims.

Potential FPC benefits: repeatable copper geometry; integrated connection tail and pads; conformal packaging; low-cost design iterations; convenient multi-gauge arrays.

Potential FPC drawbacks: large resistance footprint; copper temperature coefficient (~0.0039/°C); substrate/adhesive strain-transfer errors; copper fatigue; humidity, vibration, lead motion, and trace-width variation.

For a bonded sensor, strain transfer, adhesive cure, surface preparation, protection, and calibration matter as much as nominal trace resistance.

# 4. JLCPCB Design Rules and Explicit Assumptions

JLCPCB’s flex-PCB capabilities list 35 µm (1 oz) and 18 µm (0.5 oz) outer copper options. For 1 oz flex copper, regular minimum width/spacing is 4/4 mil (approximately 0.10/0.10 mm); for 0.5 oz, it is 3.5/3.5 mil (approximately 0.089/0.089 mm). To compare copper thickness alone, both cases below intentionally use the same 0.10 mm width and 0.10 mm spacing. These are manufacturing minimums, not preferred high-yield design targets. Copper width tolerance is listed as ±20%.

Assumptions: copper resistivity ρ = 1.68 × 10⁻⁸ Ω·m at ~20 °C; nominal width w = 0.10 mm = 1 × 10⁻⁴ m; spacing s = 0.10 mm; target R = 350 Ω; nominal copper thickness t = 35 or 18 µm. Ignore plating, etch variation, turn lengths, pads, lead resistance, and temperature for the initial ideal calculation.

# 5. Required Copper Trace Length for 350 Ω

R = ρL/(wt), therefore L = Rwt/ρ. Resistance per meter = ρ/(wt).

| Copper | R per meter | Length for 350 Ω | Ideal square active region |
| --- | --- | --- | --- |
| 35 µm (1 oz) | 4.80 Ω/m | 72.9 m | ~121 × 121 mm |
| 18 µm (0.5 oz) | 9.33 Ω/m | 37.5 m | ~87 × 87 mm |

Serpentine pitch p = w + s = 0.20 mm. For N long parallel legs of length H inside a near-square area W ≈ H, use W ≈ Np and L ≈ NH. Hence W ≈ H ≈ √(Lp). For 35 µm: √(72,900 mm × 0.20 mm) ≈ 120.8 mm. For 18 µm: √(37,500 mm × 0.20 mm) ≈ 86.6 mm. Rounded leg counts are ~604 and ~433, respectively. This first-pass model omits U-turn geometry and edge clearances, so a real layout will be larger.

Counterintuitively, thinner copper needs less length to achieve the same resistance because it has a smaller cross-sectional area. The original draft’s 20.8 m / 40.5 m values were incorrect.

# 6. Proposed FPC Geometry and Practical Alternative

Concept: one continuous serpentine copper trace on a single-layer polyimide FPC. Long sensing legs run parallel to the intended axial strain direction; rounded U-turns connect adjacent legs. Keep pads, cable tail, and strain relief outside the active region. Maintain ≥0.3 mm copper-to-outline clearance per JLCPCB and confirm bend/coverlay rules before fabrication.

Dimensioned 350 Ω concept (ideal, not fabrication-ready): 1 oz copper, 0.10 mm trace width, 0.10 mm gap, approximately 604 parallel legs, 120.8 mm active leg length, ~121 × 121 mm active region, plus turns, margins, and connection tail. The shape is a calculation target for KiCad, not a completed routing design.

A smaller proof-of-concept could target a lower resistance. For example, a 20 × 20 mm ideal active square at 0.20 mm pitch contains ~100 legs × 20 mm ≈ 2.0 m of trace. This is ~9.6 Ω at 35 µm copper or ~18.7 Ω at 18 µm copper, before turns and margins. Low-resistance bridges create higher excitation current, self-heating, and greater relative lead-resistance error, so the readout architecture must be reconsidered rather than merely increasing amplifier gain.

# 7. Readout, Calibration, and Validation

Start with a quarter-bridge prototype and a low-noise bridge readout; choose bridge completion resistors and excitation to match actual gauge resistance and power limits.

Measure the as-built trace resistance and temperature coefficient before bonding. A 350 Ω copper element changes by roughly 1.37 Ω per °C from copper TCR alone, much larger than the illustrative 0.35 Ω strain change at 500 µε.

Bond a prototype to a representative coupon or suspension member using an approved adhesive procedure; compare measured strain with a calibrated commercial foil gauge during known loading.

Repeat tests for tension, bending, temperature changes, vibration, and cable motion. Confirm whether two gauges provide bending rejection or enhancement for the intended placement; do not assume opposing sides are tension/compression under axial load.

Use suitable lead compensation (e.g., four-wire resistance measurement during characterization or bridge-specific remote sensing). Four-wire connection alone does not automatically remove every bridge error.

# 8. Questions for Initial Check-In

Is 350 Ω a strict requirement for the final device, or a calculation exercise before considering a lower-resistance copper prototype?

Which suspension member, material, and load component (axial versus bending) should be measured?

Would the team prefer a compact low-resistance FPC demonstrator or a large 350 Ω geometry in KiCad first?

# Sources and Assumptions

JLCPCB Flexible PCB Capabilities: https://jlcpcb.com/capabilities/flex-pcb-capabilities

JLCPCB Copper Weight Guide: https://jlcpcb.com/help/article/jlcpcb-copper-weight

Strain gauge overview and gauge-factor background: https://en.wikipedia.org/wiki/Strain_gauge

Copper resistivity and TCR values are standard approximate room-temperature engineering constants; actual fabricated traces must be characterized. Named university examples from the earlier draft were omitted because primary citations were not established.