Riddhi Gaddamwar — Power Electronics Onboarding

Formula Slug · 2026–2027

## Activity #1: Initial Learning — COMPLETE

Status: Finished. Notes from all required resources (Ohm’s law through PWM/RMS) are captured in the team Google Doc copy and in my working notes. I understand buck operation (Vout = D·Vin), inductor/cap roles, saturation current, and output-cap DC bias before selecting parts.

Core takeaway for Activity 2: 12 V LV bus is ~13.5 V nominal → buck must accept ~10–15 V in and regulate 5 V at ≥2 A with reasonable ripple.

## Activity #2: Buck Converter — Component Selection

Target: 12 V (13.5 V nominal) → 5 V @ ≥2 A, ~500 kHz class switching, DigiKey in-stock parts, 0603 passives where possible (LV Standards). KiCad schematic/layout deferred to Activity 3.

Design reference: MP2333 datasheet (650 kHz, synchronous buck, 3 A max). Verify inductor/cap values against the datasheet before layout.

| Component | Manufacturer P/N | DigiKey P/N | Qty | Size / package | Est. unit cost (USD) |
| --- | --- | --- | --- | --- | --- |
| Buck IC (sync, ≥2 A) | MP2333CGTL-Z | 1589-MP2333CGTL-ZCT-ND | 1 | SOT-583 (1.6×0.8 mm class) | ~$1.40 |
| Inductor 3.3 µH | CPHB322512-3R3-MGTY-73 | 445-CPHB322512-3R3-MGTY-73CT-ND | 1 | 8.0 × 6.0 mm | ~$0.45 |
| Input cap 10 µF 25 V X5R | CL21A106KAYNNNE | 1276-CL21A106KAYNNNE-ND | 1 | 0805 (1210 acceptable if stock) | ~$0.12 |
| Output cap 22 µF 6.3 V X5R | CL10A226MO7JZNC | 1276-CL10A226MO7JZNC-ND | 2 | 0603 | ~$0.20 ea |
| Feedback R top (set 5 V) | CRCW06034K99FKEA | 541-CRCW0603-4.99K-ND | 1 | 0603 0.1% | ~$0.10 |
| Feedback R bottom | CRCW06031K00FKEA | 541-CRCW0603-1.00K-ND | 1 | 0603 0.1% | ~$0.10 |
| Optional EN pull-up / SS cap | Per MP2333 datasheet | 0603 MLCC | 1–2 | 0603 | ~$0.05 |

Estimated BOM (IC + L + caps + FB): ~$2.50–3.00 per converter.

Stock check: On DigiKey, filter “In Stock” before ordering; if MP2333 is unavailable, alternate ≥2 A sync buck (e.g. TPS562201DDCR) with inductor/caps from that IC’s datasheet.

Activity 3 (KiCad schematic in pe/riddhi-buck-converter): planned next — not included in this submission.

## Extra Activity: Tractive System Shunt, BSPD Test, and Pack Voltage Sense

Full research below. Parts table updated for DigiKey-sourcable, in-stock oriented selections (March 2026). Isolated amplifiers AMC1300/AMC1311 require a high-side isolated supply (unlike integrated DC/DC parts); noted in BOM.

### Goals (summary)

Add precision shunt path alongside existing Hall sensor (cross-check + regen).

Permanent BSPD test interface per EV.8.7.4–8.7.6 (CS+/CS−, ~12.5 A equivalent via 12-turn coil on Hall).

Isolated HV pack voltage to MCU/CAN.

### Key design notes

600 A × 200 µΩ ≈ 120 mV shunt drop → use ±250 mV class isolated amp (AMC1300), not ±50 mV (AMC3302).

Pack voltage: high-value divider (MΩ) → AMC1311 (2 V max input); firmware scales Vpack.

Shunt heat at 600 A: P = I²R ≈ 72 W at 200 µΩ — thermal design required.

### Extra activity — parts (verify “In Stock” on DigiKey before order)

| Component | Manufacturer P/N | DigiKey P/N | Qty | Notes | Est. cost |
| --- | --- | --- | --- | --- | --- |
| Shunt resistor ~100–200 µΩ | Search: WSLP / shunt | Filter in stock at target R | 1 | Rating >>72 W; manganin class | $15–40 |
| Isolated current amp | AMC1300DWVR | 296-AMC1300DWVR-ND | 1 | ±250 mV in; gain 8.2; needs iso supply | ~$5–8 |
| Isolated voltage amp | AMC1311DWVR | 296-AMC1311DWVR-ND | 1 | 2 V max in; divider to ~400 V | ~$5–7 |
| Isolated DC-DC (high side) | SN6505BDBVR or similar | 595-SN6505BDBVR-ND | 1 | Powers high-side of AMC1300/1311 | ~$2–4 |
| Divider resistors 0.1% | HV chip resistors | e.g. CRCW-HP / TNPW MΩ | 2 | Sized for ≤2 V at AMC1311 input | $2–5 |
| Shunt temperature | TMP117AIDRVR | 296-TMP117AIDRVR-ND | 1 | Near shunt for derating | ~$2 |
| BSPD test jacks CS+/CS− | 4 mm shrouded banana | e.g. Keystone 9011 class | 2 | Labeled per EV.8.7.6 | $2–5 |
| Test coil wire | Magnet wire 22–24 AWG | Various | 1 roll | 12 turns through Hall aperture | ~$5 |
| Filter caps 0603 | Samsung CL / similar | 1276-CL10… | several | Per LV Standards at amp inputs | ~$1 |

## What I’m submitting now vs later

Now: Activity 1 complete; Activity 2 buck BOM; Extra tractive research + BOM.

Later: Activity 3 — KiCad buck in pe/riddhi-buck-converter, commit to riddhi-gaddamwar branch.

Riddhi Gaddamwar · Formula Slug Power Electronics